Software Download & Upgrade Instructions

Step 1: Open the zip file “upd_slseries.zip” by double clicking on it.

Step 2: Copy or drag the folder “upd_slseries” directly to the root of your flash drive. (NOTE: This is the update folder please make sure it is on the root directory of your flash drive and not inside any other folders. The DVR will only look for this “exact folder name” on your flash drive.)

Step 3: Next, you can remove the flash drive from your computer and insert it into the DVR’s usb port (front or back).

Step 4: Press the “menu” button on the front of the DVR.

Step 5: Select “Admin” for the username, then put in your password then press “Enter”. (Default password is 1234)

Step 6: Highlight System and press “Enter”.

Step 7: Highlight System/Version info and press “Enter”.

Step 8: Highlight Software upgrade then press “Enter”.

Step 9: A screen may pop up and say you need to reboot the unit first. Just press “Enter”. (After waiting a few seconds the new version number should appear on your screen)

Step 10: Change the option “No” to “Yes” next to the version number and then press “Enter” to start the update.

Step 11: The unit will now update itself and reboot once it is finished. Once the unit reboots and boots back up to its normal operating state, you may then remove the flash drive.